Electronic Fleet E85 Bid Spec

[image: image1.png]Q GASBOY





Bid Specification – Atlas Electronic Fleet E85
1. Product

1.1. Fleet non computing – Electronic register

2. Type

2.1. Dispenser – for use with remote pumping unit

3. Flowrate – capable of delivery in typical installations

3.1. Standard Flow = 15 gpm / 57 lpm 

4. Models

4.1. Single product single hose side load (9872KX) – one product unit with a single hose and nozzle boot located on the side.
4.2. Single product single hose front load (9872KX-Z) – one product unit with a single hose and nozzle boot located on the front.

4.3. Single product twin hose front load (9872KXTW1-Z) – one product unit with two hoses and nozzle boots located on the front and back of the unit.
5. Hardware Requirements

5.1. Display - Electronic Fleet: 
5.1.1. Volume only (gallons or liters) displays. 1" (2.5 cm) back lighted LCD displays. Maximum 999.000 gallons or 9999.00 liters. Displays both front and back of cabinet, except models with front load nozzle boot option which only display on the side with the corresponding nozzle boot. Last transaction maintained for more than 15 minutes by on board capacitor. Backlight with 6 Super-brite LEDs for long life.

5.2. Pulser (Electronic type) - Dual phase 1000:1 with error detection.

5.3. Meter – E-85 compatible four-piston, positive displacement flow-through C+ meter bolsters resistance to fuel contamination with self-cleaning central chamber. Electronic calibration with W&M sealable calibration switch.
5.4. Piping

5.4.1. Feedline - 1.25” steel tube, powder coated

5.4.2. Discharge - 7/8” galvanized steel tube

5.5. Filter – E-85 compatible internal 10-micron spin on filter located before meter and valve to ensure product purity and protect critical components from contamination. One filter provided per product position.
5.6. Valves - Two-stage 1” solenoid valve for preset operation that may be cleaned or replaced without breaking down pipe work.
5.7. Totalizer

5.7.1. Internal Electronic - Displayed on LCD by magnetic switch activation. Displays up to 999,999. Electronic backup – display per hose - whole units only.
5.7.2. Electro-mechanical - 7 digit non-resettable electro-mechanical totalizer – display per hose - whole units only.
5.8. Hose Hanger
5.8.1. Hose hook standard.
5.8.2. Compatible with external high spring driven cable retriever (optional).
5.9. Frame - Hot dip 13 gauge G90 galvanized steel (0.093”) with welded corners for rigidity and added strength.
5.10. Cosmetic panels - Hot dip G60 galvanized steel or stainless steel

5.10.1. Replaceable top and sides in black or stainless steel, with lower panels painted white or optional specified color or stainless steel.
5.10.2. Lower panels locked to prevent tampering
5.10.3. Paint – High quality automotive grade. Gas, oil, and UV resistant.
5.10.4. Stainless Steel: Type 304 Kooline SSt with embossed brush finish.
5.10.5. Graphics – High quality backscreened polycarbonate panels. Gas, oil and UV resistant.
5.11. Bezel and Dialface Graphics

5.11.1. Bezel: High strength structural foam for improved rigidity painted black with one-piece clear polycarbonate window. Gas, oil and UV resistant.

5.11.2. Dialface: Back screened one-piece polycarbonate overlay (black print on white background per customer spec) with cutouts as needed for displays. Gas, oil and UV resistant.

5.11.3. Brand: Replaceable polycarbonate back screened brand panel. Gas, oil and UV resistant.  Backlighted by LED light strip each side with 15 white LEDs per strip (optional).

6. Unit of Measure

6.1. US Gallons

6.2. Liters (“R”)

7. Performance

7.1. Working pressure - 50 psi rated working pressure

7.2. Meter accuracy of +/- 0.25%
8. Fuel Compatibility - Suitable for use with conventional motor fuels and oxygenated fuel blends up to 85% ethanol.

9. Electrical Requirements

9.1. 115VAC/60Hz.

9.2. 230VAC/50/60Hz

10. System Interface

10.1. Gasboy CFN RS485 interface for direct connection to control systems

10.2. Pulse Output. Selectable pulse/gallon outputs for other fuel control systems. Dual Channel Twin Hose pulse output (twin hose – two outputs per hose). Configure for pulse rates of 1:1, 10:1, 100:1, 250:1, or 500:1.
11. Inlet connection: 

11.1. Standard and High: 1-1/2” (3.8 cm) NPT.
12. Discharge: 3/4” NPT
13. Regulatory Approvals

13.1. Safety: UL or cUL approved

13.2. Weights & Measures: National Conference of Weights & Measures certified or Measurement Canada (MC) certified. Capable of sealing for measurement accuracy by local officials

13.3. FCC Part 15 Compliant
14. Warranty

14.1. 12 month standard

14.2. Optional 2 to 5 year warranty available

15. Options

15.1. FRONT LOAD NOZZLE BOOT (“Z”): Relocates standard side-oriented nozzle boot to front of cabinet for lane-oriented fueling applications. Electronic units only.

15.2. STAINLESS STEEL PANELS: Provide durable finish. All stainless steel panels ("SS"). Stainless steel front and back panels only ("SSA"). Stainless steel top and sides only ("SSTS").

15.3. TopKAT PLUS MOUNTING KIT (“D5”): Includes pump mounting hardware and Ethernet cable conduit for the TopKAT PLUS fuel management system.

15.4. LIGHT (“L”): Illuminates product panels and register area using high brightness LED strip per side with 15 LEDs per strip. Specify 115V or 230V. 

15.5. HIGH HOSE RETRACTOR (“H”) - Eases hose handling during fueling and keeps hose up and out of the way to reduce wear and run-over damage. High, post-mounted with enclosed spring return reel

16. Accessories

16.1. Hose

16.2. Breakaway

16.3. Swivel

16.4. Nozzle

17. Approved Suppliers – Gasboy Atlas 9872KX
SP-4003 - 201610
Gasboy│7300 W Friendly Ave│Greensboro, NC 27410
1


[image: image1.png]